# Recorder electronics

Electronic details

If wanted solder pin-headers to preamp and ADC boards

Using the base board to keep pin-headers straight, but without soldering to base board.

First, preamp

Then ADC board

As we only have connections for the first two audio channels, a 4-pin header is sufficient. The preamp uses pins 3,4, while the preamp of Mark (not shown here) would use, pins 1 and 2.

Connect the pins with some wire or solder as shown in next figure

Solder hydrophone header (here pico plade). To have free room for manoeuvres. Header is soldered on be back side as shown in figure.

Solder preamp to base board

Solder ADC to base board (only four pins to the right are needed)

Solder 4 wires to additional connect ADC to base board

Solder pin header to Adalogger using short headers (short pin and receptacles)

Solder female header from other side to board (having Adalogger still attached)

Remove Adalogger (for stability) and solder RTC to pin header. Mabe using the provided receptacle to keep board at 90° . Flip base board and solder pins to board.

If needed: solder DC/DC  converter with similar procedure as RTC clock (not shown in figure.

If needed: solder ELCOs

Close-up of Pre-amp ADC

# Pressure Container

Following instructions are for this configuration:

Endcap A

ID 63 mm

Tube 500 mm

OD 63 mm

Endcap C

OD 50 mm

Endcap B

ID 50 mm

OD 63 mm

Union

ID 63 mm

# Endcaps

Take 63 mm OD flat endcap B, drill 3 mm hole in center

Take endcap (endcap C) with 63 mm top and 50 mm tube, drill 3mm holes also at center

# Mounting bar

Cut piece from 10 x 10 mm bar

D

Mark holes to drill (wider correspond to spacings of drawing used for center holes.

The narower holes are seperated by half the seperation of the wider holes

# Combine mounting bar with Endcap combi

Insert into endcap combi

Fix with tape

Drill one 3 mm hole through bar and Endcap C and fix with screw before drilling the  other hole

Widen holes in bar to 4 / 4.5 mm

Resulting endcups and holes

# Adjust endcap combi for hydrophone and mounting bar

Widen the hole in the 63 mm OD endcap to 9 mm using tool to be perfectly vertical

Cut 10 mm thread into hole of 63 mm endcap

Cut 4-mm thread in tho off-center holes in 50 mm OD Endcap

Widen center hole in 50 mm OD endcap to about 24 mm diameter

# Combine electronics to flat pvc and alu bar

Tape board or stencil om pvc bar and alu bar

Drill 4 holes through pvc and alu plate

place 10x10 mm bar on flat PVC bar, smaller side touching plate

drill two 3 mm holes through bar and plate, fix again with 3mm bolt after first hole

Fix pvc plate to alu plate with 12 mm srews and three 10 mm nylon standoff an one nut

# Assemble recorder

Press mounting cap (counter end-cap) into hydrophone end-cap

Assemble mounting plate to endcap combi

Adding recorder

Place battery packs long side alu plate, place 4 double sided tapes on alu plate and fix battery packs, keeping 1 mm distance from pvc plate. Hydrophone holders may extend over 50 cm alu bar.

# Alternative mounting methods

Au U with 10 mm hex nut attached directly to hydrophone penetrator (here with Teensy 4.1) See below.

PVC bar screwed onto counter end-cap

3D printed insert replacing counter end-cap

# Construction of Alu bracket

Take an Alu U profile (20 x 10 mm)

mark a point in the middle and about 16-17 mm from the end.

Drill a 3 mm hole as precisely as possible at this point

Drill next a 10 mm hole using for example a drill head as shown in picture

Cut the Alu U profile such that the 10 mm hole is in the middle.

Fix the Alu U profile on top of the plastic bar (were the electronics will be placed)

Drill from below two 3 mm holes that are about 5 to 6 mm from the end and 4 mm from the side-

Fix the Alu profile to the plastic bar with M3 bolts.

# Gluing PV

Take endcaps and unions

# Final Recorder

Glue endcaps and unions together onto tube