# LC-MARE

Walter MX Zimmer

02-02-2026

## Objective

Aim of the low-cost marine autonomous acoustic recorder (LC-MARE) is to provide a cost-effective solution to access and record underwater sound. The implementation should be low-cost but without limitations on sound quality.

## Use cases

The envisioned use cases are defining the required characteristics of the LC-MARE autonomous recorder, and it is envisioned that LC-MARE,

- generates marine acoustic data that support

  - entry-level scientific research (e.g. for undergraduate and graduate students)

  - environmental monitoring of coastal marine areas (being protected or not)

  - marine bioacoustics in STEM education

- is used up to a water depth of

  - 100 m, if connected to surface buoy

  - 40 m, if placed on bottom by recreational scuba diver

- Is recording acoustic data with or without duty cycling

- may be calibrated to allow comparisons and exchanges of data

## Concept

The implementation should be low-cost but without constraints on quality.

There are different levels of complexity and cost factors.

- Commercially available components:

- Piezo ceramic tubes

- Programmable recording system (MCU, microSD card, RTC)

- Pressure housing components

These components are typically produced in quantities and no major cost saving is anticipated.

- Specific made components:

- Analog amplifier

- Analog-digital conversion

- Base board (to avoid cabling)

These components are specific made for LC-MARE and the ‘set-up’ costs are usually higher than the component costs. Therefore, only the production of multiple units (say 100+units) reduces the price-tag per unit to nearly the component costs.

- Custom made components:

- Hydrophone construction

- Pressure vessel construction

- Mounting frame

Depending on the use cases and individual skills these components may be build by the interested users, whereby the hydrophone construction is the most demanding task and may be appropriate only for experienced users. It is expected that, while DIY is possible, for most users the hydrophones are constructed by specialized businesses.

The construction of pressure vessels depends again on the use-cases and assuming shallow water applications (recorder depth <100 m) user build pressure vessels are feasible using pressure-rated PVC tubes and fittings.

## Implementation

### Hardware LC-Mare V2

Pre-amp

Gain 21

AD8656

25 mW

ADC

4-chan

TLV320ADC6140

50 mW

Recorder

MCU: RP2040

uSD storage

RV3028 RTC

135 mW @ fs=96 kHz

Power+Container

144 Wh Battery

720 h Endurance @210 mW

16 bar max pressure

Figure 1: LC-Mare V2 Hardware components

The components of a first LC-MARE prototype are shown in Figure 1. The electronics is composed of multiple components to facilitate easy replacement of components, if deemed necessary.

Length and diameter of the pressure tube are determined by the size of the battery holders and resulted to 500 x 63 mm for a 6 D-Cell battery pack. Using additional fittings and tubes, the power supply can easily be extended by adding additional four battery containers.

The pressure tube and fittings are 16 Bar rated PVC tubes, commercially made for water pipe constructions, e.g. for swimming pools.

### Software

The software is using standard C/C++ code and by being compatible with the Arduino infrastructure it is easily maintained, modified and downloaded to the microcontroller of the LC-MARE. The few parameters that are expected to be modified at sea may be provided using a terminal program, a python GUI or by a file on the microSD card that can be prepared prior to the at-sea activity.

# Recorder

Figure 3: LC-Mare August 2024 version

## Implementation details

The LC-MARE implementation will be open source and hard and software implementation will be published at GitHub - WMXZ-EU/LC-MARE: low-cost marine acoustic recorder

### Hydrophone

Hydrophone consists of three components: Cylindrical piezo-ceramic tube, BlueRobotics penetrator, and polyurethane rubber. The hydrophone is at some distance from the penetrator to reduce acoustic interference of the pressure container. The use of BlueRobotics penetrators facilitates the changes of hydrophones and pressure containers.

### Pressure Container

Endcap

ID 63 mm

Tube 500 mm

OD 63 mm

Endcap

OD 50 mm

Endcap

ID 50 mm

OD 63 mm

Union

ID 63 mm

Figure 2: Pressure container components

The pressure container consists of 5 components a 500 mm long tube, an union, 3 end-caps with different diameters. To connect the hydrophone a 10 mm hole will be drilled in the middle-sized end-cap. The PVC-U components are rated with PN16 and should withstand 16 Bar internal pressure. It is expected that they also withstand 16 Bar external pressure, which corresponds to a water depth 150 m. The PVC-U tubes and fittings are purchased from an online store (pcv-welt.de) but may be found in hardware stores.

### Mounting system

The electronic and battery packs are mounted onto an aluminium plate (35x50x2) by means of double-sided adhesive tapes. To separate electronics from conducting aluminium an additional PVC plate is added. The mounting plate is attached to hydrophone penetrator using an aluminium U frame. All components are from the local hardware store.

### Electronics

For the electronics we have following components

#### Pre-amplifier

The pre-amplifier is a differential-to-differential voltage amplifier, the Spice simulation is shown next:

Gain is controlled effectively by the resistor R1. There is still some gain increase at 400 kHz, that can be removed by increasing the feedback capacitors C3, C7 somewhat. The selected OP-AMPS are of CMOS type with 2.7 nV/sqrt(Hz) noise voltage each.

A more lower noise pre-amplifier with FET and ultra-low noise bipolar amplifier would be a possible alternative and is in development.

#### Analog Digital Conversion

The ADC is based on the Texas Instruments TLV320ADC6140 chip and implements the circuitry of the datasheet.

The ADC features 4 differential input channels each of which can be amplified up to 40 dB. A dedicated LDO regulator provides 3.3V for the analog circuitry.

Sampling frequencies can be up to 768 kHz for a single channel, 384 kHz for two channels and 192 kHz for all four channels. Bit depth is 24 bit.

#### Putting it all together

The final electronics board features 1 pre-amp, a 4-channel ADC, an Adafruit Feather Adalogger Rp2040 and a RV3028 RTC.

## Software

Fully programmed with C/C++ code but can be compiled and downloaded within the Arduino ecosystem or using Makefiles.

All software is available on GitHub.

Data storage is using ExFat filesystem and after moving the disk, the data can be accessed directly by the PC. This allows also to store/edit configuration files on the disk ahead of closing the pressure container.

### GUI

The following Python based GUI shows some of the parameters that may be retrieved and stored and may be the basis for a Custom GUI.

This GUI shows some parameters that are used for controlling the functionality of LC-MARE. Gray-ed fields are de-activated as not in use in the actual implementation.

The Form will be populated with the ‘Load’ button. The ‘Sync’ button will synchronize the RTC with the PC time. Parameters will be downloaded to MCU using the ‘Save’ button.

The start button will start the acquisition process and will display the messages from the MCU. At the same time the button changes to a Stop button to stop the acquisition process.

The Restart button will reboot the connected MCU. If the h_start value is zero, then the MCU will reboot immediately.  Any value >0 will delay the reboot to the next full hour. For example, for a value of 1 the MCU will reboot  at the next full hour, with a value of 5 the MCU will reboot at the last full hour in the next 5 hours.

Parameters shown in GUI:

- fsamp:	sampling frequency in Hz

- proc: 	online processing: 0 wav file; 1: integer loss-less compression

- shift: 	in case of compression indicates the number of lower bits that are to be ignored

- again: 	analog gain in dB in ADC

- t_acq: 	length of files in s

- t_on: 	length of acquisition block in s

- t_rep: 	time in s between starts of acquisition blocks in s (0 means continuously)

- h_1: 	start hour of first acquisition period

- h_2: 	end of first acquisition period

- h_3: 	start of second acquisition period

- h_4: 	end of second acquisition period

- h_start: 	reboot delay in hours (0 immediately, 1 next full hour, etc)

In above GUI image, the acquisition timing parameters are set for continuous recording